MODULE 2

G1 Arm Planning, Collision Avoidance,
and ROS 2 Execution Wrapper

Technical implementation, validation, file inventory, handoff guide, and operating procedure

Current status Software planning chain validated for both arms, including point-to-pose and complete grasp sequences. ROS 2 wrapper created with execution disabled by default. Physical robot readiness still requires controller, TF, gripper, simulation, and low-speed hardware validation.

Prepared for project handoff and orchestrator integration
Document date: 12 October 2026

# Contents

1. Executive Summary

2. Module 2 Scope and Architecture

3. Forward and Inverse Kinematics

4. Collision Model and Motion Planning

5. plan_to_pose() Interface

6. plan_grasp_from_point() Interface

7. Validation Results

8. Required Files and Project Layout

9. ROS 2 Wrapper Architecture

10. ROS 2 Installation and Configuration

11. ROS Service Usage

12. Execution Safety and Failure Handling

13. Orchestrator Handoff Contract

14. Robot-Readiness Checklist

15. Troubleshooting

Appendix A. Commands

Appendix B. References

Document use Use Sections 8-13 as the group handoff guide. Use Section 14 as the gate before enabling real robot motion.

# 1. Executive Summary

Module 2 provides collision-aware arm planning for the Unitree G1 23-DOF configuration using separate five-joint left and right arm models. The implementation combines learned inverse kinematics, deterministic forward-kinematics verification, four-dimensional precomputed collision masks, edge-checked A*, full five-joint trajectory validation, adaptive grasp staging, and a ROS 2 service/execution wrapper.

| Capability | Status | Evidence |
| --- | --- | --- |
| Forward kinematics | Complete | Scalar, vectorized, and differentiable FK implemented and tested. |
| Hybrid learned IK | Complete | 1,000,000 training samples per arm; runtime FK guard applied. |
| Static collision masks | Complete | 15,959,240 states per arm; both manifests complete. |
| Collision-aware point-to-pose | Complete | Left and right tests reached exact verified goals with zero failed dense waypoints. |
| Complete grasp sequence | Complete in software | Both arms returned successful to_pregrasp, approach, lift, and retract segments. |
| ROS 2 wrapper | Implemented | Planning services, trajectory execution, stop service, stale-state checks, and execution gate. |
| Physical robot readiness | Pending validation | Requires live controller, TF, gripper, simulation, stop, and low-speed hardware testing. |

Safety position A successful plan is not authorization to move hardware. The execution wrapper keeps allow_execution=false by default and requires fresh measured joint states before sending any trajectory.

# 2. Module 2 Scope and Architecture

## 2.1 Responsibilities

Receive a Cartesian pose or object point in the torso_link frame.

Generate and verify five arm-joint goals.

Plan collision-aware trajectories from measured current joints.

Return structured success or failure results with no executable partial path on failure.

Optionally execute validated paths through a ROS 2 FollowJointTrajectory controller.

Coordinate grasp sequencing while leaving gripper control at defined orchestration points.

## 2.2 End-to-end flow

Cartesian target / object point
  -> request validation
  -> hybrid neural IK
  -> forward-kinematics verification
  -> collision-aware numerical IK fallback when required
  -> static collision mask lookup
  -> nearest-free grid goal and exact-goal bridge
  -> edge-checked 4D A*
  -> dense five-joint collision validation
  -> structured plan
  -> optional ROS 2 FollowJointTrajectory execution

## 2.3 Coordinate and joint conventions

| Item | Convention |
| --- | --- |
| Base frame | torso_link |
| Pose vector | [x, y, z, rx, rz] |
| Position units | metres |
| Angle units | radians |
| Arm joint order | ShoulderPitch, ShoulderRoll, ShoulderYaw, Elbow, WristRoll |
| Grid joints | First four joints; WristRoll is restored and checked continuously |
| Dense waypoint spacing | No joint step greater than 1 degree |

# 3. Forward and Inverse Kinematics

## 3.1 Arm geometry

The configuration file defines the left and right kinematic chains, joint origins, axes, lower and upper limits, grasp-frame transforms, training workspace, grasp geometry, and collision allowances. All production components read the same YAML configuration to avoid mismatched geometry.

## 3.2 Learned IK training

| Dataset split | Per arm | Both arms |
| --- | --- | --- |
| Training | 1,000,000 | 2,000,000 |
| Validation | 100,000 | 200,000 |
| Held-out test | 100,000 | 200,000 |
| Total examples | 1,200,000 | 2,400,000 |

The final models use a hybrid FK-based loss. Validation and test samples were not used to update network weights.

| Metric | Left arm | Right arm |
| --- | --- | --- |
| Mean position error | 4.37 mm | 4.17 mm |
| P95 position error | 8.43 mm | 8.43 mm |
| Held-out predictions below 10 mm | 97.63% | 97.52% |

## 3.3 Runtime verification

Reject targets outside the configured training workspace.

Predict a five-joint IK solution.

Run deterministic FK on the predicted joints.

Require position error <= 0.01 m, rx error <= 0.10 rad, and rz error <= 0.15 rad.

Return no path if verification fails.

## 3.4 Collision-aware IK fallback

A pose-accurate neural IK branch can still collide. When the exact neural IK goal collides, collision_aware_ik.py performs bounded multi-start numerical optimization, applies the same pose thresholds, rejects colliding branches, and returns a collision-free alternative when one exists.

# 4. Collision Model and Motion Planning

## 4.1 Collision geometry

| Moving geometry | Representation |
| --- | --- |
| Shoulder pitch | URDF cylinder |
| Shoulder roll | URDF cylinder |
| Shoulder yaw | STL mesh |
| Elbow | STL mesh |
| Hand | STL mesh |

| Static body geometry | Files |
| --- | --- |
| Torso and head | torso_link.STL, head_link.STL |
| Waist and pelvis | waist_yaw_link.STL, waist_support_link.STL, pelvis.STL |

The only non-zero permitted mesh penetration is shoulder_yaw to torso at 0.001 m. All other configured collision pairs use zero allowance.

## 4.2 Mask generation

| Mask | Shape | States | Blocked | Blocked fraction |
| --- | --- | --- | --- | --- |
| Left | 83 x 55 x 76 x 46 | 15,959,240 | 7,103,787 | 44.51% |
| Right | 83 x 55 x 76 x 46 | 15,959,240 | 6,820,214 | 42.74% |

## 4.3 Planner behavior

Validate exact five-joint start and IK goal.

Map start and goal to the 4-degree grid.

If the rounded goal cell is blocked, search nearby free cells.

Validate a dense bridge from each candidate cell to the exact IK goal.

Run A* while sampling every grid edge at no more than 1-degree increments.

Restore WristRoll and the exact continuous IK goal.

Revalidate the complete dense five-joint path before success.

Why edge checking matters Two free 4-degree grid nodes can still have a collision between them. edge_checked_astar.py rejects such edges during search rather than detecting the collision only after a complete route is produced.

# 5. plan_to_pose() Interface

## 5.1 Python call

from module2_planner import Module2Planner

planner = Module2Planner()
result = planner.plan_to_pose(
    arm='left',
    current_joints=[q1, q2, q3, q4, q5],
    target_pose=[x, y, z, rx, rz],
)

if not result.success:
    # Hard failure: execute nothing.
    raise RuntimeError(result.message)

joint_path = result.joint_path

## 5.2 Result contract

| Field | Meaning |
| --- | --- |
| success | True only after IK, planning, and dense collision validation pass. |
| message | Human-readable success or failure reason. |
| joint_names | Ordered five-joint names for the selected arm. |
| joint_path | Dense Nx5 path in radians. |
| verified_goal_joints | Exact FK-verified IK solution. |
| position_error_m / rx_error_rad / rz_error_rad | Runtime verification measurements. |
| selected_grid_state | Free A* cell used to bridge to the exact target. |
| exact_goal_cell_blocked | Indicates whether rounded target cell was blocked. |
| nodes_expanded | A* search diagnostic. |

## 5.3 Validated examples

| Arm | Target pose | Waypoints | Position error | Result |
| --- | --- | --- | --- | --- |
| Left | [0.30, 0.15, 0.00, 0, 0] | 26 | 4.64 mm | Success |
| Right | [0.30, -0.15, 0.00, 0, 0] | 53 | 3.46 mm | Success; rounded goal cell blocked and bridged |

# 6. plan_grasp_from_point() Interface

## 6.1 Segment sequence

to_pregrasp
  -> orchestrator opens gripper
approach
  -> orchestrator closes gripper
lift
retract

## 6.2 Python call

from module2_grasp_planner import Module2GraspPlanner

grasp_planner = Module2GraspPlanner()
result = grasp_planner.plan_grasp_from_point(
    arm='right',
    current_joints=[q1, q2, q3, q4, q5],
    object_xyz=[object_x, object_y, object_z],
)

if result.success:
    for name in ['to_pregrasp', 'approach', 'lift', 'retract']:
        path = result.segments[name]['joint_path']

## 6.3 Adaptive staging

The actual grasp pose remains fixed. Pre-grasp and retract are flexible staging poses. The planner searches nearby position and yaw candidates while retaining the same safety and FK thresholds.

| Stage | Candidate variation |
| --- | --- |
| Pre-grasp | Z: 0/+3/+6/+9 cm; X: 0/+4/-3 cm; Y: 0/+4/-4 cm; yaw: 0/±15/±30/±45 degrees |
| Retract | Z: 0/+3/-3 cm; X: 0/+4/-4/+8 cm; Y: 0/+4/-4 cm; yaw: 0/±15/±30/±45 degrees |

## 6.4 Validated grasp outcome

| Arm | Pre-grasp yaw | Retract yaw | Complete segments | Result |
| --- | --- | --- | --- | --- |
| Left | -0.5248 rad | 0 rad | 4 | Success |
| Right | -0.2606 rad | 0 rad | 4 | Success |

# 7. Validation Results

## 7.1 Test evidence

Collision backend import and CollisionManager creation passed.

All audited STL meshes existed and loaded with plausible metre-scale dimensions.

Collision classification correctly treated sub-1 mm shoulder mounting overlap as non-blocking.

Both 15.96 million-state masks completed and contained meaningful mixtures of blocked and free states.

Left and right edge-checked point-to-pose plans succeeded, ended at exact IK goals, and had zero failed dense waypoints.

Invalid workspace request returned success=false and an empty path.

Adaptive grasp tests passed, and both complete grasp plans returned success=true.

## 7.2 Selected unit tests

| Test file | Purpose |
| --- | --- |
| test_forward_kinematics.py | FK correctness and arm-chain behavior. |
| test_torch_fk.py | Differentiable TorchG1FK agreement from scripts/torch_g1_fk.py. |
| test_collision_allowance.py | Pair-specific shoulder mounting allowance. |
| test_edge_checked_astar.py | Intermediate edge collision detection. |
| test_singleton_path.py | Preserve exact continuous start and goal inside one grid cell. |
| test_module2_planner_contract.py | Reject malformed point-to-pose requests safely. |
| test_grasp_geometry_integration.py | Use canonical grasp geometry. |
| test_grasp_uses_plan_to_pose.py | Route every Cartesian waypoint through the unified safety pipeline. |
| test_adaptive_staging.py | Generate workspace-bounded position and yaw staging candidates. |

# 8. Required Files and Project Layout

## 8.1 Essential runtime files

| Path / file | Purpose | Required at runtime |
| --- | --- | --- |
| config/arm_kinematics.yaml | Authoritative kinematics, limits, workspace, grasp, and collision configuration. | Yes |
| models_hybrid_1m/g1_left_hybrid_best.pt | Left hybrid IK checkpoint. | Yes |
| models_hybrid_1m/g1_right_hybrid_best.pt | Right hybrid IK checkpoint. | Yes |
| collision_masks/left_static_collision_mask.npy | Left 4D collision mask. | Yes |
| collision_masks/right_static_collision_mask.npy | Right 4D collision mask. | Yes |
| meshes/*.STL | Body and arm collision meshes. | Yes |
| scripts/module2_planner.py | Single point-to-pose API. | Yes |
| scripts/module2_grasp_planner.py | Complete segmented grasp API. | Yes |
| scripts/guarded_ik_solver.py | Neural IK loading and FK verification. | Yes |
| scripts/collision_aware_ik.py | Alternative collision-free IK branch search. | Yes |
| scripts/collision_aware_planner.py | Nearest-free bridge and complete collision-aware planning. | Yes |
| scripts/edge_checked_astar.py | Dense edge validation during A*. | Yes |
| scripts/joint_space_astar.py | Grid and path conversion utilities. | Yes |
| scripts/collision_mask.py | Memory-mapped Boolean mask interface. | Yes |
| scripts/g1_collision_probe.py | FCL collision object construction and pair classification. | Yes |
| scripts/g1_forward_kinematics.py | Deterministic pose reconstruction. | Yes |
| scripts/grasp_geometry.py | Canonical grasp and pre-grasp geometry. | Yes |

Filename clarification: scripts/torch_g1_fk.py defines TorchG1FK, the differentiable PyTorch FK used by scripts/train_ik_hybrid.py. Runtime FK verification uses scripts/g1_forward_kinematics.py. Keep torch_g1_fk.py in the repository for training reproducibility and maintenance.

## 8.2 Generation and diagnostic files

| File | Purpose |
| --- | --- |
| scripts/generate_static_collision_mask.py | Generate or resume complete left/right masks. |
| scripts/audit_collision_meshes.py | Verify required STL files and dimensions. |
| scripts/validate_dense_collision_path.py | Independent validation of a saved Nx5 path. |
| scripts/diagnose_collision_aware_ik.py | Broader numerical IK branch search for a selected pose. |
| scripts/train_ik_hybrid.py | Train hybrid IK checkpoints. |
| scripts/generate_ik_dataset*.py | Generate training data. |

## 8.3 Recommended layout

arm_planning/
├── config/arm_kinematics.yaml
├── scripts/
│   ├── module2_planner.py
│   ├── module2_grasp_planner.py
│   ├── guarded_ik_solver.py
│   ├── collision_aware_ik.py
│   ├── collision_aware_planner.py
│   ├── edge_checked_astar.py
│   ├── joint_space_astar.py
│   ├── collision_mask.py
│   ├── g1_collision_probe.py
│   ├── g1_forward_kinematics.py
│   └── grasp_geometry.py
├── models_hybrid_1m/
├── collision_masks/
├── meshes/
├── urdf/
└── tests/

# 9. ROS 2 Wrapper Architecture

## 9.1 Packages

| Package | Build type | Contents |
| --- | --- | --- |
| module2_interfaces | ament_cmake | PlanToPose.srv, PlanGraspFromPoint.srv, StopExecution.srv |
| module2_ros | ament_python | Execution node, YAML configuration, launch file |

## 9.2 Services

| Service | Purpose |
| --- | --- |
| /module2/plan_to_pose | Plan and optionally execute one Cartesian target. |
| /module2/plan_grasp_from_point | Plan and optionally execute segmented grasp sequence. |
| /module2/stop | Request cancellation of active FollowJointTrajectory goal. |

## 9.3 Controller integration

The wrapper sends validated paths through control_msgs/action/FollowJointTrajectory. The action supports path tolerances, goal tolerances, invalid-joint reporting, rejection, cancellation, and controller error results. Controller names are parameters because the simulator or G1 bridge may use different action paths.

Controller assumption The wrapper does not publish Unitree /lowcmd directly. If the selected G1 stack lacks FollowJointTrajectory, create a tested adapter to the Unitree arm SDK instead of bypassing Module 2 safety gates.

## 9.4 Execution safety

allow_execution is false by default.

Every request requires fresh measured joint states.

Request current_joints must match measured state within start_tolerance_rad.

Each trajectory segment must begin near the measured state.

Only result.success=true paths may be executed.

Only one trajectory execution is allowed through the node at a time.

Controller path and goal tolerances are supplied.

The stop service requests action cancellation.

Gripper services can be mandatory for hardware grasp execution.

# 10. ROS 2 Installation and Configuration

## 10.1 Workspace setup

cd ~/ros2_ws/src
# Extract/copy both packages here:
# module2_interfaces
# module2_ros

cd ~/ros2_ws
source /opt/ros/humble/setup.bash
colcon build --packages-select module2_interfaces module2_ros
source install/setup.bash
export PYTHONPATH=/absolute/path/to/arm_planning/scripts:$PYTHONPATH

## 10.2 Configuration file

Edit module2_ros/config/module2.yaml before launch. Set an absolute project_root and verify every checkpoint, mask, controller, topic, and gripper endpoint.

module2_execution_node:
  ros__parameters:
    project_root: /absolute/path/to/arm_planning
    joint_state_topic: /joint_states
    left_trajectory_action: /left_arm_controller/follow_joint_trajectory
    right_trajectory_action: /right_arm_controller/follow_joint_trajectory
    open_gripper_service: ''
    close_gripper_service: ''
    allow_execution: false
    require_gripper_services: false
    joint_state_timeout_sec: 0.5
    start_tolerance_rad: 0.05
    path_tolerance_rad: 0.10
    goal_tolerance_rad: 0.05

## 10.3 Planning-only launch

ros2 launch module2_ros module2.launch.py

The expected initial mode is planning only. Keep allow_execution=false until Section 14 is satisfied.

# 11. ROS Service Usage

## 11.1 Point-to-pose request

ros2 service call /module2/plan_to_pose module2_interfaces/srv/PlanToPose \
"{arm: left, current_joints: [0,0,0,0,0], target_pose: [0.30,0.15,0.00,0.0,0.0], execute: false}"

## 11.2 Grasp request

ros2 service call /module2/plan_grasp_from_point module2_interfaces/srv/PlanGraspFromPoint \
"{arm: right, current_joints: [0,0,0,0,0], object_xyz: [0.30,-0.15,0.00], execute: false}"

## 11.3 Stop request

ros2 service call /module2/stop module2_interfaces/srv/StopExecution "{}"

## 11.4 Reconstruct flattened paths

import numpy as np

path = np.asarray(response.flattened_joint_path).reshape(
    response.waypoint_count, 5
)

offset = 0
segments = {}
for name, count in zip(response.segment_names, response.segment_waypoint_counts):
    length = count * 5
    segments[name] = np.asarray(
        response.flattened_segment_paths[offset:offset + length]
    ).reshape(count, 5)
    offset += length

# 12. Execution Safety and Failure Handling

## 12.1 Hard-failure rule

Mandatory orchestrator rule If success is false, the orchestrator must execute nothing. An empty path or missing segment is never treated as a recoverable command.

## 12.2 Common rejection conditions

Unknown arm name.

Malformed, non-finite, or out-of-limit joints.

Target outside trained workspace.

Neural and numerical IK unable to meet FK thresholds.

Exact IK goal colliding.

No nearby free goal or collision-free bridge.

No edge-checked A* route.

Dense path collision.

Missing or stale ROS joint state.

Request start differs from measured state.

Trajectory controller unavailable or rejects goal.

Path or goal tolerance violation.

Required gripper service unavailable.

## 12.3 Execution timing

The wrapper assigns monotonically increasing time_from_start values using seconds_per_waypoint and a minimum final duration. Final timing should be tuned in simulation to the actual controller, speed limits, and mechanical capability. Do not increase speed as a workaround for controller tracking errors.

# 13. Orchestrator Handoff Contract

## 13.1 Startup

Start or connect to the robot state publisher and trajectory controllers.

Start Module 2 once; models and masks are loaded at startup.

Confirm the service names and planning-only behavior.

Read current joints from the same live source consumed by Module 2.

## 13.2 Point-to-pose orchestration

request execute=false
  -> if failure: stop workflow and report message
  -> if success: inspect/approve returned path
  -> request execute=true only after hardware enablement gate

## 13.3 Grasp orchestration

plan_grasp_from_point(execute=false)
  -> verify success and four segments
  -> execute to_pregrasp
  -> open gripper
  -> execute approach
  -> close gripper
  -> execute lift
  -> execute retract

## 13.4 Data contract

| Data | Requirement |
| --- | --- |
| Arm | left or right |
| Current joints | Five finite radians in configured joint order; must match measured state |
| Target pose | [x, y, z, rx, rz] in torso_link |
| Object point | [x, y, z] in torso_link |
| Output joint path | Dense Nx5 radians |
| Failure behavior | success=false and no executable motion |

# 14. Robot-Readiness Checklist

## 14.1 Required before enabling execution

☐ Confirm ROS distribution and Python/runtime compatibility on Linux.

☐ Confirm /joint_states or configured equivalent publishes all ten relevant arm joints with matching names.

☐ Confirm left and right FollowJointTrajectory action names with ros2 action list.

☐ Validate joint ordering expected by each controller.

☐ Compare Python FK against simulator or robot TF at zero, bent, and task configurations.

☐ Verify grasp-frame offsets physically or in the authoritative simulator.

☐ Run planning-only ROS requests using live measured states.

☐ Play both arm paths in simulation and inspect body, table, object, and opposite-arm clearance.

☐ Test /module2/stop cancellation.

☐ Configure and test gripper open/close services.

☐ Keep emergency stop accessible.

☐ Perform one-arm, reduced-speed, cleared-workspace tests.

☐ Test pre-grasp and approach without object contact.

☐ Only then set allow_execution=true.

## 14.2 Known limitations

Static masks include robot body and selected arm self-collisions, but not the table, bottle, live obstacles, or the opposite moving arm.

The wrapper assumes a FollowJointTrajectory controller. Unitree low-level interfaces require a separate adapter.

Current completion is software validation, not certification of safe physical operation.

Adaptive staging may select long paths; controller timing and practical motion efficiency need simulation tuning.

# 15. Troubleshooting

| Symptom | Likely cause | Action |
| --- | --- | --- |
| Missing Module 2 assets | Incorrect project_root or file layout | Verify YAML, checkpoints, masks, and meshes. |
| Joint state is missing or stale | Wrong topic or timeout too strict | Confirm topic, names, publisher rate, and joint_state_timeout_sec. |
| Current joints do not match measured state | Caller used stale start values | Use the latest measured state; do not bypass the check. |
| Trajectory action unavailable | Wrong controller name or controller not active | Inspect ros2 action list and controller manager. |
| INVALID_JOINTS | Controller joint names/order differ | Match YAML joint names and controller configuration. |
| PATH_TOLERANCE_VIOLATED | Controller cannot track requested path | Lower speed, tune controller, verify hardware state; do not widen blindly. |
| Exact IK goal is colliding | Pose uses colliding branch or infeasible target | Allow numerical fallback or change staging pose, not collision limits. |
| Planning succeeds but execution disabled | allow_execution remains false | This is expected until readiness gates are passed. |
| Gripper skipped | No service configured and optional mode active | Configure services and set require_gripper_services=true for hardware. |

# Appendix A. Key Commands

## A.1 Python validation

python -m pytest -v
python scripts\audit_collision_meshes.py
python scripts\validate_dense_collision_path.py right_collision_aware_path.npy --arm right

## A.2 Mask status

type collision_masks\left_static_collision_state.json
type collision_masks\right_static_collision_state.json

## A.3 ROS inspection

ros2 topic list
ros2 action list
ros2 service list
ros2 topic echo /joint_states --once

# Appendix B. References

ROS 2 control_msgs FollowJointTrajectory action documentation: https://docs.ros.org/en/jazzy/p/control_msgs/action/FollowJointTrajectory.html

ROS 2 joint_trajectory_controller documentation: https://control.ros.org/jazzy/doc/ros2_controllers/joint_trajectory_controller/doc/userdoc.html

ROS 2 custom message and service interface tutorial: https://docs.ros.org/en/humble/Tutorials/Beginner-Client-Libraries/Custom-ROS2-Interfaces.html

Unitree ROS 2 repository: https://github.com/unitreerobotics/unitree_ros2

Project-specific quantitative results, filenames, service designs, and validation outcomes in this document are based on the completed Module 2 implementation and command outputs produced during development.